ANTI ALGAE COPPER ELECTROLYZER

An ideal and effective method against (blanket)weed. The Copper Electrolyzer is installed before the pond return inlet. The pond water flows through the Copper Electrolyzer unit. A copper unit, or Electrolyzer, is positioned within the housing, and the copper unit discharges a minimal amount of copper ions into the water flowing through the housing. These copper ions (Cu2+) in the water have a positive charge and attach to and destroy the cell walls of algae, bacteria, viruses and other primitive organisms. As the cell wall is thereby damaged, these organisms are no longer able to take up any nutrients, and are therefore unable to multiply. The process of applying a positive charge to copper is called copper electrolysis. Inlet/outlet: Ø50 mm.

PLEASE NOTE: (too much) copper is harmful for fish!

Pending EU regulations or conditional approval, national regulations apply to the trade and use of copper-containing products. The copper used in these units complies with the REACH registration. It is the sole responsibility of the buyer to inform himself/herself about applicable local regulations regarding the use of (and trade in) copper for water disinfection.

| Item No. | Type | Size | Max. Dosage |
| --- | --- | --- | --- |
| 3300029 | Copper Electrolyzer | 50 mm | 0,7 ppm |

ANTI ALGAE COPPER ELECTROLYZER SPARE PARTS

| 33000029 | Xclear Anti Algae Copper Electrolyzer |  |
| --- | --- | --- |
| B290076 | Control Box for Copper Electrolyzer | B |
| 3300027 | Housing Xclear Anti Algae Copper Electrolyzer | E |
| B290029 | Copper v unit for Blue Lagoon UV-C Ionizer | A |
| 3901045 | Nut 70mmx33.5mm BLACK | K |
| E801509 | O-Ring for 3-way connection (1 piece) | G |
| 3902068 | O-ring 56x3 UV-C housing/glass SIL | C |
| B220013 | Nut ABS black | F |
| E801136 | Glue socket 50mm transparent ABS | H |